This file (READCDA2.DOC) describes the READCDA utility (version 2).

Index:

- What is READCDA2 ?

- What CD-ROM drives can be used ?

- How to use READCDA

- READCDA and Windows 3.xx/Windows 95

- Timing data

- Revision history

- List of CD-ROM drive that do/do-not support reading digital audio

- Credits

What is READCDA ?

==========================================================================

The READCDA program enables you to use your CD-ROM drive to read Digital

Audio Data from a Compact Disc. It reads the data digitally, so there is

no loss in quality.

What CD-ROM drives can be used ?

==========================================================================

Unfortunately not all CD-ROM drives support the drive call that enables

READCDA to read the data. I wrote this program and made it to work on a

Panasonic double speed drive (CR-562B, Creative Labs 2x drive). It also

works on various Sony and Teac drives.

!!!!!! It does NOT work on Mitsumi drives !!!!!

Thats Mitsumi's fault. They think the copyright issues could cause them

problems ...

Since we are on that subject, i take no responsibility for the use of

this program !!!

There are many CD-ROM drives.

On the end of this file is a list of CD-ROM drives that can be used.

How to use READCDA:

==========================================================================

The READCDA program can be used in two different ways:

1) By starting it without parameters. It will prompt for them.

2) By starting it with parameters. It will start directly (providing you

specified all the required parameters).

The possible parameters are shown when you start the READCDA program

with an unknown option (like -help ;-)).

It will display the following usage screen:

Usage: readcda [options]

If no options are specified the user is asked for the required data

Options: -file <output-filename>   [needed]

-track <tracknumber>      which track to read

-from <location>          from where to read

-length <length>          how long to read

Note: -track or -from are needed, and -from must be combined with -length

where -track CAN be combined with -length

both -length and -from can be 'min:sec' or sector number

Extra options:

-debug <level> Display more output with higher level

-wav           Write an WAV file (=default)

-cdda          Write the RAW CDDA output

-nbuf <#buf>   Use #buf buffers (default 20x18K), for better memory usage

-swapbytes     Swap all bytes (MSB/LSB) in output

-swapwords     Swap all words (channels) in output

-nosynch       Skip the synchronization algorithm

-synch         Keep using the synchronization algorithm

Normal usage with parameters would be (example):

readcda -track 4 -length 0:10 -file some.wav

or

readcda -from 05:23 -length 0:10 -file some.wav

The options change the way READCDA works:

-debug

Will add extra debug output

-wav (default)

Will write a WAV output file, playable in Windows

-cdda

Will write RAW digital output

-nbuf (default is determined automatically, 18K each)

Changes the number of buffers used for reading the data.

Those buffer are filled, and written to the output file.

Then the data in the last buffer is re-read plus the rest of the buffers

The larger the number of buffers the better performance, since 1 in

nbuf buffers will we read twice.

The READCDA program try and reserve as much buffers as possible.

If the program won't work properly you can try to run it with less

buffers.

-swapbytes

Swaps all the bytes read from the CD. One CD-ROM drive (a HP4020i)

has been reported to need this.

-swapwords

Somebody claimed that the Left and Right channel were swapped.

I do not beleive so, but this option will do the job.

-nosynch

READCDA will tell you if synchronization is not needed. You can

then speed up the program by adding the '-nosynch' switch.

You can also use this switch to measure what speed you drive is

capable of reading digital audio.

-synch

Causes READCDA to USE the synchronization algorithm.

READCDA and Windows 3.xx/Windows 95

==========================================================================

Since READCDA is a dos program, it will not use Windows when you run

it in Windows. But many people might run it in Windows anyway, since

that is their normal environment.

A few remarks concerning the use of READCDA from within Windows.

- If it fails to operate like it should (resulting in synchronization

errors) try disabling any SMARTDRV or other caching on your CD-ROM

drive. You can do this for smartdrv with 'SMARTDRV D-' or whatever

drive your CD-ROM is...

- Using READCDA from withing Windows 95 is NOT straightforward. Since

READCDA does work in Windows 3 you would expect it to work in

Windows 95. But is doesn't !!! You can solve the problem by

installing the old (16-bit) device driver and the MSCDEX (which IS

supplied with Windows 95) in CONFIG.SYS and AUTOEXEC.BAT. When you

do that READCDA works again. Somehow Microsoft did not make the

newer CDFS device driver compatible with Windows 3.xx plus the

CD-ROM device driver/MSCDEX.

Timing data:

==========================================================================

This feature is put in by Stewart Addison. His description:

The program measures the speed of the drive when reading DA. The final

output of the program will show the reading speed with a line such as:

Reading speed was somewhere between 0.94x and 1.23x

Two values are given for the reading speed. The lower value is

calculated by timing the overall execution time. This will include the

time taken to write the output to the hard drive, so unless your hard

drive is extremely fast, this will be lower than the actual CD-DA

reading speed.

The second value is calculated from the time the program spends in the

ReadLong function, which is the function which actually performs the DA

extraction. In theory this only takes into account the speed of the

CD-ROM drive so this figure should be accurate. However, it can be

higher than the true value, since the CD-ROM drive may cache the reads

while the program is writing the output, so the subsequent ReadLong will

be faster.

The above output is from a drive which performs DA extraction at single

speed - the true value is 1.00x. Depending on your system, when writing

the output to a file and performing synchronization you may get results

lower than the true DA reading speed.

You can usually obtain an accurate speed measurement by sending the

output to the NUL device, and performing a read of at least one minute.

The command line:

readcda -file NUL -track 1 -length 01:00

Should give you an output with both speeds virtually identical.

If you also specify the -nosynch commandline option than the

theoretical maximum reading speed of the CD-ROM drive is measured.

I've tried it on four drives, and all give different results:

My own Pioneer DR-UA124X : Single speed with or without synchronization.

0% Synchronization errors

An NEC273 : Reads dual speed with -nosynch, a bit less (~1.6x) with

synchronization. Usually <10% sync. errors

A Goldstar GCD-R540B : Reads at single speed with -nosynch, slower with

sync. 100% synchronization errors (all the time)

3*Panasonic 563B : Like the NEC for speed, but usually gives 100%

errors - 2 were on 386/40s, the third on a DX/2-66

Revision history:

==========================================================================

READCDA was written in august 1994.

It was based upon a simple source by Yeng-Chee Su, found on the Net.

To that source I added a synchronization algorithm, that prevented

problems with not so fast drives and/or systems.

It did not contain major bugs, so it remained unchanged for a long

time. In the meantime Michiel Overtoom wrote a Windows program based

upon the source of READCDA, which has a much better user interface.

But a few people came with some wishes and there were a few small

bugs, so I decided to write a version 2.

It is now August 1996. Version 2 is here !

It contains:

- The buffers are only half of what they were resulting in 10-20%

better execution time, since there is less data read twice.

- Some bugfixes in the synchronization algorithm

- Commandline parameters

- Timing data is given after the reading process (Thanks Stewart!)

- Commandline parameters for controlling some (extra) features

Drives that do/do-not support reading digital audio:

==========================================================================

The list below contains some drives that do or do not support reading

digital audio. This list is far from complete however. For a better

list please look at the WEB pages about reading digital audio on:

http://www.tardis.ed.ac.uk/~psyche/pc/cdrom/CDDA.html

IDE-like CD-ROM drives that do support the reading of digital audio:

- Sony CDU31, CDU33, CDU55, CDU76 , others probably

- Panasonic/Creative Labs drives (I think all of them)

- Toshiba XM-5302B

- Teac 4x

- Hitachi

- Pioneer DR-UA124X

SCSI CD-ROM players that work (maybe not with DIDO or READCDA, but

some other programs do support SCSI CD-ROM drives, like DA2WAV and

CDINFO):

- Apple 300, 300i, 300e (Sony CDU-8003, CDU-8003A)

- Apple CD300+

- Chinon CDS-535

- NEC CDR 200, 300, 400, 500, 600, 900 series

- NEC CDR 84-1/74-1 (with special firmware) (yet to be verified)

- Plextor (formerly TEXEL) DM-3028, DM-5028

- Sony CDU-561

- Toshiba XM3301 (Silicon Graphics)

- Toshiba XM3401, XM4101, XM3501

Drives that do NOT support reading digital audio:

- Mitsumi

- Yamaha

- Aztech

- Goldstar (Some newer ones DO support it)

Credits:

==========================================================================

The first one to thank is Yeng-Chee Sy (yenchee@csie.nctu.edu.tw) for

his initial source. I found it somewhere in a newsgroup on usenet.

After making it this little program definitely proved the

possibilities of first usenet and later the internet.

I get about one e-mail every two weeks, and that about 2 years after

writing the first version of READCDA !!!

So thanks to everybody that came with suggestions.

Special thanks to:

- Michiel Overtoom <motoom@xs4all.nl> for writing the nice DIDO

program for Windows (See http://www.xs4all.nl/~motoom ).

- Bodo Wolf <Bodo@Wolf.Ping.de> for is input for commandline parameters.

- Stewart Addison <psyche@tardis.ed.ac.uk> for putting the timing

stuff in READCDA and for finding some obscure bugs.

Hopefully i did not forgot any of you ...

If you have any questions, don't hesitate to mail me.

Also checkout the WEB pages about CD-DA from Stewart Addison:

http://www.tardis.ed.ac.uk/~psyche/pc/cdrom/CDDA.html

Klaas Hemstra <hst@mh.nl>